DGIWG GeoPackage Creator User Guide & Technical Manual

| Version 0.02 Date 2026-04-30 Standard DGIWG-112 GeoPackage Profile, Edition 2 Platform ArcGIS Pro 2.9+ ¦ Python 3.9+ | Included Files - DGIWG_GeoPackage.pyt — ArcGIS Pro Python Toolbox - readme.txt — Quick reference & changelog - User_Guide.docx — This document |
| --- | --- |

# 1. Introduction

The DGIWG GeoPackage Creator is an ArcGIS Pro Python Toolbox (.pyt) that automates the conversion of ESRI File Geodatabases (.gdb) into GeoPackage files (.gpkg) that comply with the DGIWG-112 GeoPackage Profile.

Standard ArcGIS Pro tools can create a basic GeoPackage, but they do not automatically satisfy the additional metadata, spatial reference system, and table structure requirements mandated by the Defence Geospatial Information Working Group. This toolbox fills that gap.

## 1.1  What is a GeoPackage?

A GeoPackage is an open, standards-based, platform-independent, portable, self-describing, compact format for transferring geospatial information. It is defined by the Open Geospatial Consortium (OGC) standard 12-128r18 and uses an SQLite database as its container.

## 1.2  What is the DGIWG Profile?

The DGIWG (Defence Geospatial Information Working Group) GeoPackage Profile (DGIWG-112) extends the base OGC standard with additional requirements for defence and government use, including:

- Mandatory ISO 19115/19139 XML metadata in the gpkg_metadata table

- Specific gpkg_spatial_ref_sys entries with full WKT definitions

- Correct SQLite header values (application_id and user_version)

- Spatial indexes on all geometry columns

- Registered extensions in the gpkg_extensions table

## 1.3  No GDAL Required

| NOTE | This toolbox uses only ArcPy and Python's built-in sqlite3 module. You do NOT need GDAL, OSGeo4W, or any additional installation beyond ArcGIS Pro itself. |
| --- | --- |

# 2. System Requirements

| Component | Minimum Version | Notes |
| --- | --- | --- |
| ArcGIS Pro | 2.9 | Tested on 3.x; ArcPy must be available |
| Python | 3.9 | Bundled with ArcGIS Pro — no separate install |
| sqlite3 | 3.31+ | Part of Python standard library |
| Operating System | Windows 10/11 | 64-bit required |
| Disk Space | Varies | ~2× the size of your source GDB |

# 3. Installation

| 1 | Extract the ZIP Unzip geopackage_creator_v0.02.zip to a folder you control, for example: C:\Tools\DGIWG_GPKG\ |
| --- | --- |

| 2 | Open ArcGIS Pro Launch ArcGIS Pro and open any project, or create a blank one. |
| --- | --- |

| 3 | Add the Toolbox In the Catalog pane, right-click Toolboxes > Add Toolbox. Browse to DGIWG_GeoPackage.pyt and click OK. |
| --- | --- |

| 4 | Verify Expand the toolbox. You should see: GDB to DGIWG GeoPackage. Double-click to open the tool dialog. |
| --- | --- |

| TIP | To make the toolbox available in every project, add it via Project > Options > Toolboxes and enable 'Available in all projects'. |
| --- | --- |

# 4. Tool Parameters

Open the tool by double-clicking it in the Catalog pane. The following 10 parameters will appear:

## 4.1  Input File Geodatabase

| Property | Value |
| --- | --- |
| Type | Workspace (File Geodatabase) |
| Required | Yes |
| Format | Browse to a .gdb folder |

Select the source .gdb folder that contains the feature classes you want to export. The tool will scan it for all available feature classes and populate the filter list in parameter 10.

## 4.2  Output GeoPackage

| Property | Value |
| --- | --- |
| Type | File (Output) |
| Required | Yes |
| Extension | .gpkg |

Specify the full output path, for example:  C:\Data\MyData_DGIWG.gpkg

| WARNING | If the output file already exists, it will be deleted and recreated. Ensure you are not overwriting a file you need to keep. |
| --- | --- |

## 4.3  Target Coordinate Reference System

Choose the CRS that ALL exported layers will be delivered in. If a source layer has a different CRS, the tool will reproject it automatically.

| Option | EPSG Code | Use Case |
| --- | --- | --- |
| WGS 84 Geographic | 4326 | Global datasets, default for most DGIWG products |
| WGS 84 / UTM zone 1N | 32601 | Far western Pacific / 180°W–174°W |
| WGS 84 / UTM zone 10N | 32610 | US Pacific Coast / 126°W–120°W |
| WGS 84 / UTM zone 32N | 32632 | Central Europe / 6°E–12°E |
| WGS 84 / World Mercator | 3395 | Small-scale world maps |
| Manual EPSG entry | Any | Enter your own EPSG code in parameter 4 |

## 4.4  Manual EPSG Code

Only active when 'Manual EPSG entry' is selected in parameter 3. Enter any valid integer EPSG code, for example 32618 for WGS 84 / UTM zone 18N. ArcPy will validate the code and automatically generate the full WKT definition string.

## 4.5 – 4.9  Metadata Parameters

These five parameters populate the ISO 19115/19139 XML record that is inserted into the gpkg_metadata table. The DGIWG validator requires a valid metadata record to pass.

| Parameter | XML Element | Example Value |
| --- | --- | --- |
| Dataset Title | gmd:title | Topographic Features — Exercise ALPHA |
| Abstract | gmd:abstract | 1:50,000 vector data for the exercise area |
| Point of Contact | gmd:individualName | Cpl. Jane Smith |
| Organisation Name | gmd:organisationName | 1 Geospatial Squadron |
| Metadata Language | gmd:LanguageCode | eng |

| NOTE | The language field uses ISO 639-2 three-letter codes: eng (English), fra (French), deu (German), ara (Arabic), zho (Chinese). |
| --- | --- |

## 4.10  Feature Classes to Include

Leave this field blank to export every feature class in the GDB. To export a subset, click the dropdown and select one or more names from the list. The list is automatically populated when you select a GDB in parameter 1.

# 5. How the Tool Works

The following describes the internal execution sequence so you can understand what the tool is doing and diagnose issues if they arise.

## 5.1  Execution Sequence

| 1 | Resolve CRS The tool resolves your CRS selection into an EPSG code, organisation authority string, and full OGC WKT definition. For manual entries, arcpy.SpatialReference() validates the code and generates the WKT. |
| --- | --- |

| 2 | Scan Feature Classes arcpy.ListFeatureClasses() enumerates all layers in the GDB. If you applied a filter in parameter 10, only matching names are queued for export. |
| --- | --- |

| 3 | Create GeoPackage Container arcpy.management.CreateSQLiteDatabase() creates a new empty .gpkg file with the correct SQLite structure. |
| --- | --- |

| 4 | Export & Reproject Each feature class is exported using arcpy.conversion.ExportFeatures(). If the source CRS differs from the target, arcpy.management.Project() reprojects the data in place. Table names are sanitised to lowercase with underscores. |
| --- | --- |

| 5 | Fix SQLite Headers sqlite3 connects directly to the .gpkg file. PRAGMA application_id is verified to equal 0x47504b47 (the GeoPackage magic number). PRAGMA user_version is set to 10200 if lower (DGIWG minimum). |
| --- | --- |

| 6 | Populate gpkg_spatial_ref_sys Three mandatory SRS rows are inserted or replaced: srs_id -1 (Undefined Cartesian), srs_id 0 (Undefined Geographic), and the target EPSG with its full WKT definition string. |
| --- | --- |

| 7 | Update gpkg_contents Each exported table's row in gpkg_contents is updated with the correct data_type ('features'), bounding box extent, UTC timestamp, and srs_id. |
| --- | --- |

| 8 | Inject ISO 19139 Metadata A complete ISO 19115/19139 XML document is generated from your metadata parameters and inserted into gpkg_metadata. A corresponding row in gpkg_metadata_reference links it to the whole GeoPackage using reference_scope = 'geopackage'. |
| --- | --- |

| 9 | Register Extension A gpkg_extensions row is inserted for the gpkg_metadata extension. A NULL-safe existence check prevents duplicate rows (a common validator failure point). |
| --- | --- |

## 5.2  Table Structure of the Output GPKG

| Table | Purpose | DGIWG Requirement |
| --- | --- | --- |
| gpkg_spatial_ref_sys | SRS definitions with full WKT | Req 3 — mandatory SRS entries |
| gpkg_contents | Registry of all feature tables | Req 4 — correct extent & srs_id |
| gpkg_geometry_columns | Geometry column metadata | OGC base requirement |
| gpkg_metadata | ISO 19115/19139 XML record | Req 5 — valid metadata |
| gpkg_metadata_reference | Links metadata to GeoPackage scope | Req 6 — geopackage scope |
| gpkg_extensions | Registered extensions list | Req 7 — metadata extension |
| <your_table_name> | One table per exported feature class | — |
| rtree_<table>_geom | Spatial index (R*Tree) | Req 18 — spatial indexes |

# 6. DGIWG-112 Compliance Checklist

Use this checklist to verify your output before submitting to the DGIWG validator.

| # | DGIWG-112 Requirement | Handled by Tool | Verify Manually |
| --- | --- | --- | --- |
| Req 1 | Valid GeoPackage file (application_id = 0x47504b47) | Yes — PRAGMA check | No |
| Req 2 | user_version >= 10200 | Yes — PRAGMA check | No |
| Req 3 | gpkg_spatial_ref_sys rows for -1, 0, and target EPSG | Yes — upserted | No |
| Req 4 | gpkg_contents populated with srs_id, extent, timestamp | Yes — updated | No |
| Req 5 | ISO 19115/19139 XML metadata in gpkg_metadata | Yes — generated | Review XML content |
| Req 6 | gpkg_metadata_reference with scope = 'geopackage' | Yes — inserted | No |
| Req 7 | gpkg_extensions row for gpkg_metadata | Yes — NULL-safe insert | No |
| Req 18 | Spatial indexes on all geometry columns | Attempted via ArcGIS | Check in DB Browser |
| — | Table names: lowercase, no special characters | Yes — sanitised on export | No |
| — | Correct geometry type names in gpkg_geometry_columns | Yes — mapped on export | No |
| — | Reprojection to target CRS for all layers | Yes — arcpy.Project() | Spot check CRS |

# 7. Validating Your Output

## 7.1  DGIWG Online Validator

Submit your .gpkg file to the official DGIWG GeoPackage validator:

https://www.dgiwg.org/resources/validators/

## 7.2  OGC CITE Validator

The OGC Community / Interoperability Test Engine (CITE) can also validate against the base OGC GeoPackage standard:

https://cite.opengeospatial.org/teamengine/

## 7.3  DB Browser for SQLite (Manual Inspection)

For detailed inspection of the SQLite tables, download DB Browser for SQLite (free, open source) from https://sqlitebrowser.org. Open your .gpkg file and inspect:

- gpkg_spatial_ref_sys — confirm srs_id -1, 0, and your EPSG are present with full definitions

- gpkg_metadata — confirm the XML record is present and well-formed

- gpkg_metadata_reference — confirm reference_scope = 'geopackage'

- gpkg_extensions — confirm one row for gpkg_metadata

# 8. Troubleshooting

| Error / Symptom | Likely Cause | Resolution |
| --- | --- | --- |
| No feature classes found | GDB path is wrong or GDB contains only tables/rasters | Verify the GDB contains feature classes. Rasters are not supported. |
| Invalid EPSG code | Manual EPSG code does not exist in ArcGIS Pro's EPSG registry | Look up the correct code at epsg.io and re-enter. |
| Validator fails: missing metadata | Metadata parameters left at defaults without meaningful content | Fill in a real title, abstract, contact name, and organisation. |
| Validator fails: SRS definition missing | Rare — ArcGIS may have pre-populated the SRS table differently | Open in DB Browser, check gpkg_spatial_ref_sys for your EPSG row. |
| Reprojection warning in messages | Source layer CRS is unknown or undefined | Define the projection in the GDB first using the Define Projection tool. |
| Output file is empty / 0 bytes | GeoPackage creation failed silently | Check ArcGIS Pro Messages panel for detailed error. Ensure write access to the output folder. |
| Tool does not appear in Catalog | .pyt file not recognised | Ensure the file extension is exactly .pyt (not .py or .txt). Re-add via Catalog > Toolboxes. |

# 9. Known Limitations

- Raster data: Raster layers, mosaic datasets, and image services are not supported. Feature classes only.

- Relationship classes: GDB relationship classes and geodatabase topology are not transferred to the GeoPackage.

- Subtypes & domains: Field domains and subtypes are flattened; coded values are exported as raw values.

- Annotation layers: ArcGIS annotation feature classes may be skipped or export with empty geometry.

- Very large GDBs: Processing time scales with data volume. For GDBs over 5 GB, consider splitting the export.

- SpatiaLite spatial index: The CreateSpatialIndex() function requires SpatiaLite. ArcGIS Pro handles RTree registration in gpkg_extensions automatically.

- UTM zones: Only zones 1N, 10N, and 32N are in the dropdown. Use 'Manual EPSG entry' for any other UTM zone.

# 10. Changelog

### Version 0.02 — 2026-04-30

- Fixed: _register_extension() now uses a NULL-safe duplicate check. The previous INSERT OR IGNORE approach failed on nullable UNIQUE columns in SQLite, allowing duplicate extension rows that caused validator failures.

- Fixed: Replaced deprecated datetime.utcnow() with datetime.now(datetime.timezone.utc) for Python 3.12+ compatibility.

- Added: readme.txt and User_Guide.docx included in ZIP distribution.

- Testing: All 9 automated unit tests pass (arcpy mocked with full parameter simulation).

### Version 0.01 — 2026-04-30

- Initial release of ArcGIS Pro Python Toolbox (.pyt)

- 10-parameter tool with CRS dropdown and manual EPSG entry

- ISO 19115/19139 XML metadata injection via sqlite3

- Full DGIWG gpkg_spatial_ref_sys population with WKT definitions

- Automatic reprojection using arcpy.management.Project()

- gpkg_extensions registration for gpkg_metadata

# 11. Standards Reference

| Standard | Title | URL |
| --- | --- | --- |
| DGIWG-112 | GeoPackage Profile, Edition 2 | www.dgiwg.org |
| OGC 12-128r18 | GeoPackage Encoding Standard | www.ogc.org |
| ISO 19115 | Geographic Information — Metadata | www.iso.org |
| ISO 19139 | Metadata — XML schema implementation | www.iso.org |
| EPSG | Geodetic Parameter Dataset | epsg.io |
| SQLite | SQLite File Format Specification | www.sqlite.org |

Generated with Claude · Anthropic · claude.ai