DGIWG GeoPackage Compliance Validator

User Manual

Version 1.59

Automated conformance checker for DGIWG STD-DP-19-005 v1.1

(GeoPackage Profile 1.4, Edition 1.1)

Mapping and Charting Establishment

Geomatics Engineering Trials & Evaluation Support Section (GETESS)

GPL-2.0-or-later  |  © 2026 Eui Soo SON

# Contents

1. Introduction

1.1 Who this is for

1.2 What the tool decides, and what it does not

2. What's New in v1.59

2.1 Reachable top verdict (Req 4)

2.2 Req 18 now reports whether XSD validation ran

2.3 --quiet is now actually quiet

2.4 Internal robustness

2.5 Release packaging

2.6 Carried over from v1.58

3. System Requirements & Installation

3.1 Required

3.2 Set up a dedicated environment (Anaconda Prompt)

3.3 Optional libraries

3.4 Folder layout

4. Running the Validator

4.1 Interactive (file picker)

4.2 Command line — single file or folder

4.3 As a Python module

4.4 Complete option reference

4.5 Exit codes

5. Output Reports

6. Understanding the Results

6.1 Per-requirement status

6.2 Overall file verdict

6.3 Confidence levels (roll-up)

6.4 Cascade root-cause advisory

6.5 Source-software forensics

7. Requirements Coverage

8. Manual Confirmation

9. Verifying Your Installation

9.1 Running the self-test (Anaconda Prompt)

9.2 What the v1.59 self-test covers

9.3 Building the release archive

10. Known Limitations

11. Support

# 1. Introduction

The DGIWG GeoPackage Compliance Validator is a Python tool that automatically checks a GeoPackage (.gpkg) file against the requirements of DGIWG STD-DP-19-005 v1.1 (the DGIWG GeoPackage Profile 1.4, Edition 1.1). It evaluates the file against the profile's 37 numbered requirements, grades each one, and produces detailed per-file and roll-up reports.

The validator is read-only. The GeoPackage is opened through a SQLite read-only URI, so the file being inspected cannot be modified by this tool under any circumstances. All checks run locally against the SQLite database inside the GeoPackage; a small number of optional cross-checks use the internet or bundled offline reference data when available.

## 1.1 Who this is for

Geomatics technicians, data producers, and quality-assurance staff who need to confirm that a GeoPackage deliverable conforms to the DGIWG profile before it is released or accepted.

## 1.2 What the tool decides, and what it does not

Of the 37 requirements, 34 are evaluated automatically. Three are not, and cannot be:

- Req 1 and Req 2 (OGC base conformance) require the OGC CITE TeamEngine test suites. They are always reported as SKIPPED.

- Req 6 (conditional extensions) depends on which DGIWG product profile applies to the deliverable, which is context this tool does not have. It is always reported as SKIPPED.

Important.  A clean verdict from this tool is a statement about the 34 requirements it can test. Full conformance to STD-DP-19-005 v1.1 additionally requires the three checks above to be carried out separately. Since v1.59 every HTML report states this directly beneath the verdict, so the caveat travels with the report.

The tool also never repairs a file. Where a FAIL is reported, the correction must be made by the data producer in their own software and the validator re-run.

# 2. What's New in v1.59

v1.59 is the first full release of the 1.5x line; v1.58 was a pre-release. The changes below are behavioural — they alter what the tool reports, not only how it words it — so read this section before comparing v1.59 output against results produced by an earlier build.

## 2.1 Reachable top verdict (Req 4)

Req 4 (Optional Extensions) previously returned PASS* in every case, including when a GeoPackage registered no optional extensions at all. Because the CONFORMANT verdict requires zero PASS* results, that single behaviour made CONFORMANT unreachable for every file ever tested. Req 4 now distinguishes the two cases:

- No optional extension registered — PASS. Nothing optional is claimed, so there is nothing for a human to confirm.

- One or more registered — PASS*, with the list of extensions and a prompt to confirm each is permitted by the applicable product profile.

Optional extensions still cannot produce FAIL; that is true by definition and unchanged.

## 2.2 Req 18 now reports whether XSD validation ran

Req 18 validates DMF metadata documents. Its deepest stage — XSD structural validation against the bundled DMF schema, which catches wrong element order and missing mandatory children — requires lxml. When lxml was absent, that stage was skipped silently. The same GeoPackage could therefore report PASS on one workstation and FAIL on another, with nothing in either report to explain the difference.

Req 18 now always states the outcome of the XSD stage in its detail text, and a metadata document that passed only the field-level checks is reported as PASS* rather than PASS. lxml has also been added to the startup optional-library probe, so a missing lxml is visible before the run rather than invisible after it.

## 2.3 --quiet is now actually quiet

The report-writing routines printed “HTML report written”, “Rollup HTML written” and “Rollup CSV written” unconditionally, so --quiet still produced three extra lines per batch plus one per file. All three now honour the flag, and --quiet delivers exactly one summary line per file — which is what makes it usable piped into a QA log.

## 2.4 Internal robustness

- _manual_checks() returned a mix of 3-tuples and 4-tuples depending on whether a check succeeded or errored. Every entry is now a uniform 4-tuple, removing a latent ValueError from any consumer that unpacked the result.

- Result-dictionary consumers now select requirement rows by key type rather than by a hardcoded list of internal key names. Adding a new internal key can no longer crash report rendering with a type error.

- Two unused network helpers imported by the checks module were removed.

- The command-line examples in --help referenced “dgiwg_validator.py”, a filename that does not exist in the distribution. They now show the versioned launcher and the module form, and cover --quiet, --recursive, --output-dir and --fail-fast.

## 2.5 Release packaging

- package_release.py now verifies a manifest of required assets before it builds. A missing user manual, quick-start page or launcher aborts the build instead of producing a quietly incomplete archive.

- Superseded user manuals are excluded from the staged folder, so only the current version ships.

- The archive is named DGIWG_GeoPackage_Validator_v1.59.zip — the “_pre” suffix is gone — and VERSION.txt reads “(release)”.

- QUICKSTART.html is new: a single self-contained page covering what the tool is, setup, the three ways to run it, how to read a verdict, and the flags worth knowing.

## 2.6 Carried over from v1.58

v1.58 corrected the Req 1/2/15 identifiers against Table 6 of the v1.1 standard, fixed a Req 26 false PASS on 512-pixel tiles, made an absent gpkg_extensions table non-fatal for Req 3/4/5, restored the EPSG REST API datum lookup in Req 13, and corrected the Req 10 guidance text to match Table 13. Those fixes are all present in v1.59.

# 3. System Requirements & Installation

## 3.1 Required

- Python 3.9 or newer. The core checks use only the Python standard library.

- Windows, Linux or macOS. No GDAL installation is required.

## 3.2 Set up a dedicated environment (Anaconda Prompt)

Never install into the base environment.  Installing these packages into Anaconda's base environment is the most common cause of dependency conflicts and unexplained runtime failures. Always create and activate a dedicated environment first.

conda create -n dgiwg python=3.11 -y
conda activate dgiwg
pip install shapely Pillow pyproj lxml

## 3.3 Optional libraries

These enable the deepest automated checks. When a library is absent the affected requirement degrades to PASS* (a partial pass) rather than failing, and the report names the missing library on the affected row.

| Library | Enables | Install command |
| --- | --- | --- |
| pyproj | Req 13 — datum-name cross-check against the EPSG registry | pip install pyproj |
| shapely | Req 24 — OGC geometry validity sampling | pip install shapely |
| Pillow | Req 26 — decoding tile image BLOBs to confirm 256×256 pixels | pip install Pillow |
| lxml | Req 18 — DMF metadata XSD structural validation (new in v1.59) | pip install lxml |

tkinter (normally bundled with Python) provides the optional file-picker window. If it is missing, pass file or folder paths on the command line instead — no other functionality is affected.

## 3.4 Folder layout

Keep the launcher script and the dgiwg_validator package together in the same folder:

DGIWG_GeoPackage_Validator_v1.59\
    DGIWG_Validator_v1_59.py        launcher (double-click or run with Python)
    QUICKSTART.html                 five-minute quick-start page
    DGIWG_GeoPackage_Validator_User_Manual_v1.59.docx
    run_local_tests.py              self-test harness
    package_release.py              release packaging script
    dgiwg_epsg_cache.json           bundled offline EPSG reference data
    VERSION.txt
    dgiwg_validator\                the package (checks, constants, reporting)

If you move the launcher  away from the dgiwg_validator folder, Python will report ModuleNotFoundError: dgiwg_validator. Keep the two together, or run the tool as a module with python -m dgiwg_validator.

# 4. Running the Validator

## 4.1 Interactive (file picker)

Double-click DGIWG_Validator_v1_59.py, or run it with no arguments. A window opens where you can select one or more .gpkg files.

python DGIWG_Validator_v1_59.py

## 4.2 Command line — single file or folder

python DGIWG_Validator_v1_59.py  "C:\data\my_file.gpkg"
python DGIWG_Validator_v1_59.py  "C:\data\gpkg_folder"

A folder is scanned for .gpkg files at the top level; add --recursive to include subfolders. Files named *.gpkg.gpkg are ignored as double-extension artifacts.

## 4.3 As a Python module

python -m dgiwg_validator  "C:\data\my_file.gpkg"
python -m dgiwg_validator  --offline --quiet --output-dir "C:\QA\reports" "C:\data"

## 4.4 Complete option reference

| Flag | Effect |
| --- | --- |
| FILE_OR_FOLDER | One or more .gpkg files or folders, given as positional arguments. Omit to open the interactive file picker. |
| --file FILE | Explicit single file. Identical to a positional argument. May be repeated. |
| --dir FOLDER | Explicit folder. Identical to a positional argument. May be repeated. |
| --recursive | Search folders recursively for .gpkg files instead of top level only. |
| --offline | Disable all internet checks (EPSG URI verification, OGC TMS validation). Affected checks return PASS* with an explicit offline-mode note rather than a network-error note. Use on air-gapped networks. |
| --no-install | Suppress the optional-library install prompts. Libraries already present are still used; missing ones produce PASS* results as normal. Use on locked-down systems. |
| --output-dir FOLDER | Write all HTML, JSON and CSV reports to this folder instead of the default reports\ subfolder beside each GeoPackage. |
| --quiet | Suppress per-file progress banners. Prints one summary line per file. Since v1.59 this also suppresses the report-written lines. |
| --fail-fast | Stop after the first file with any FAIL result, write the roll-up for what has been processed, and exit with code 1. Useful in scripted acceptance gates. |
| --sample-size N | Number of geometry/tile BLOBs sampled per table for Req 24 and Req 26. Defaults: 25 geometries per table, 5 tile BLOBs per zoom level. Higher values broaden coverage at the cost of speed. |
| --timeout SECONDS | Network timeout for all HTTP checks. Default 8 seconds. Raise on slow links; lower to fail faster against unresponsive endpoints. |
| --json | Retained for backward compatibility only. JSON companion reports have been written unconditionally since v1.53. |
| --version | Print the version and exit. |
| -h, --help | Print the full option list with worked examples and exit. |

## 4.5 Exit codes

| Code | Meaning |
| --- | --- |
| 0 | The run completed. Note that this does not mean the files were conformant — read the verdict. |
| 1 | --fail-fast was given and a non-conformant file was encountered. |

# 5. Output Reports

For each GeoPackage the validator writes a detailed report and a JSON companion; when several files are processed it also writes a combined roll-up. By default these land in a reports\ subfolder beside the GeoPackage.

| Output file | Description |
| --- | --- |
| <name>_DGIWG_Report.html | Per-file report: every requirement, its status, the supporting evidence, the probable cause, and manual-confirmation steps. Start here. |
| <name>_DGIWG_Report.json | Machine-readable per-file results: schema_version, verdict, counts, detected source software, per-requirement status and detail, internet-check outcomes, and the cascade root-cause note. |
| DGIWG_GPKG_FINAL_REPORT.html | Roll-up across all files: per-file summary with links, a file × requirement matrix, and a requirement-by-requirement confidence table. |
| DGIWG_GPKG_FINAL_REPORT.csv | Roll-up summary in spreadsheet form. |

The report filename carries a source-software tag derived from the forensic analysis — _QGIS, _ARCGIS or _UNKNOWN. Where two GeoPackages from different folders share a basename, a numeric suffix is appended so no report overwrites another.

Read-only target folders  are handled: if reports cannot be written beside the GeoPackage, they are written to a temporary folder and the console prints the path used.

# 6. Understanding the Results

## 6.1 Per-requirement status

| Status | Meaning |
| --- | --- |
| PASS | The requirement was fully checked and met. |
| PASS* | Partial pass. Structural or presence checks passed, but the content could not be fully verified automatically — an optional library is missing, the run was offline, or the requirement is not fully automatable. Review the Manual Confirmation panel. |
| FAIL | The requirement was checked and not met. The detail text names the offending table, value or CRS. |
| SKIPPED | Not applicable to this file (for example a vector-CRS check on a tiles-only file), or the check needs external context — Req 6 needs the product profile, Req 1 and 2 need OGC CITE TeamEngine. |

## 6.2 Overall file verdict

| Verdict | Condition |
| --- | --- |
| CONFORMANT | No FAIL and no PASS* results. Every automatable requirement was fully verified. |
| LIKELY CONFORMANT (partial checks) | No FAIL, but one or more PASS* results remain to be confirmed manually. |
| NON-CONFORMANT | One or more FAIL results. |

On an air-gapped network,  LIKELY CONFORMANT is the normal best outcome. --offline caps the CRS checks that would otherwise query the EPSG registry at PASS*, because they genuinely cannot be confirmed without network access. This is not a defect in the file being tested.

Remember that all three verdicts describe the 34 automatable requirements only — see section 1.2.

## 6.3 Confidence levels (roll-up)

The roll-up assigns each requirement a confidence rating: High (fully automated and verifiable), Medium (partial check or one known gap), and Low (cannot be automated, or no applicable files were present in the run). A FAIL or SKIPPED row is always shown as Low confidence, because in those cases there is no automated confirmation to be confident about.

## 6.4 Cascade root-cause advisory

When gpkg_metadata or gpkg_metadata_reference is absent, a single root cause produces FAIL results across many metadata requirements at once. The validator detects this pattern and prints an advisory banner naming the missing table and the requirements it accounts for, so that eight failures are not mistaken for eight separate problems.

## 6.5 Source-software forensics

Before running the requirement checks, the validator fingerprints the GeoPackage to identify the software that produced it — QGIS/GDAL, ArcGIS, or UNKNOWN — from schema traces, extension registrations and metadata patterns. The detection signals are listed in the report, and where the detected software has a known behaviour relevant to a failed requirement, an audit hint is attached to that requirement explaining the likely origin of the finding.

# 7. Requirements Coverage

The validator evaluates all 37 DGIWG requirements. Type M = Mandatory, C = Conditional. Names and identifiers below are read directly from the validator's own requirement table and match Table 6 of STD-DP-19-005 v1.1.

Requirements 1, 2 and 6 are reported as SKIPPED by design — see section 1.2.

| # | Requirement | Identifier | Type |
| --- | --- | --- | --- |
| 1 | GeoPackage Base definition | /req/geopackage/base | M |
| 2 | GeoPackage Options definition | /req/geopackage/options | M |
| 3 | Mandatory Extensions | /req/extensions/mandatory | M |
| 4 | Optional Extensions | /req/extensions/optional | C |
| 5 | Extensions Not Allowed | /req/extensions/not-allowed | M |
| 6 | Conditional Extensions | /req/extensions/conditional | C |
| 7 | Raster CRS Allowed | /req/crs/raster-allowed | M |
| 8 | CRS Raster Tile Matrix Set | /req/crs/raster-tile-matrix-set | M |
| 9 | 2D Vector CRS | /req/crs/2d-vector | M |
| 10 | 3D Vector CRS | /req/crs/3d-vector | M |
| 11 | Gridded 2D CRS | /req/crs/gridded-crs-2d-allowed | M |
| 12 | Gridded 3D CRS | /req/crs/gridded-crs-3d-allowed | M |
| 13 | WKT for CRS | /req/crs/wkt | M |
| 14 | Compound CRS Usage | /req/crs/compound | C |
| 15 | Compound CRS WKT | /req/crs/crs-compound-wkt | M |
| 16 | Gridded Compound CRS WKT | /req/crs/gridded-crs-compound-wkt | C |
| 17 | Gridded CRS Epoch WKT | /req/crs/gridded-crs-epoch-wkt | C |
| 18 | GeoPackage Metadata DMF | /req/metadata/dmf | M |
| 19 | GeoPackage Metadata Document | /req/metadata/gpkg | M |
| 20 | Complete Row Metadata | /req/metadata/row | M |
| 21 | User Row Metadata | /req/metadata/user | C |
| 22 | Product Metadata | /req/metadata/product | C |
| 23 | Product Partial Metadata | /req/metadata/product-partial | C |
| 24 | Data Validity | /req/validity/data-validity | M |
| 25 | Tile Matrix Width/Height | /req/tile/size-matrix | M |
| 26 | Tile Pyramid Data Size | /req/tile/size-data | M |
| 27 | Zoom Level Factor | /req/zoom/factor | M |
| 28 | Multiple Zoom Matrix Sets | /req/zoom/matrix-sets-multiple | C |
| 29 | Single Zoom Matrix Set | /req/zoom/matrix-sets-one | C |
| 30 | Tile Matrix Set CRS BBox | /req/bbox/crs | M |
| 31 | Tile Layer Metadata | /req/metadata/tile | C |
| 32 | Feature Layer Metadata | /req/metadata/feature | C |
| 33 | Gridded Extension Core | /req/gridded/gridded-coverage-extension-core | C |
| 34 | Gridded Field Name Elevation | /req/gridded/field-name-elevation | C |
| 35 | Gridded Center Value Elevation | /req/gridded/center-value-elevation | C |
| 36 | Gridded Zoom Factor | /req/gridded/zoom-factor | C |
| 37 | Gridded User Row Metadata | /req/gridded/metadata-gridded-user | C |

# 8. Manual Confirmation

Each FAIL or PASS* result in the per-file HTML report includes a Manual Confirmation panel: what to check, how to check it in DB Browser for SQLite, and a ready-to-run SQL query. Use these to independently verify the tool's findings before accepting or rejecting a deliverable.

For PASS* rows the report also shows a Manual Confirmed count — for example “3 / 3” — recording how many items a deeper database check was able to confirm. A full count means the partial pass has been substantiated as far as the database allows; a partial or zero count marks the rows that most need a human.

Because the tool is read-only, any corrective action — removing a disallowed extension, fixing a CRS definition, regenerating a tile pyramid — must be performed by the producer in their own software before the validator is re-run.

# 9. Verifying Your Installation

A self-test harness ships with the tool. It generates three synthetic GeoPackages using only the Python standard library — one conformant tile pyramid, one carrying more than ten deliberate violations, and one feature dataset with an invalid geometry — runs the validator against them in offline mode, and compares every produced requirement status against an expectation table that adjusts automatically for whichever optional libraries are installed.

## 9.1 Running the self-test (Anaconda Prompt)

conda activate dgiwg
cd C:\Users\Son\Documents\DGIWG\DGIWG_GeoPackage_Validator_v1.59
python run_local_tests.py

Expected on the final lines:

RESULT: <n>/<n> assertions passed
ALL TESTS PASSED

A detailed step-by-step log named local_test_log_<timestamp>.txt is written next to the script, whether the run passes or fails. If anything fails, send that log file for analysis — it records the generated test data, the exact command lines used, the full validator output, and the expected-versus-actual status for every assertion.

## 9.2 What the v1.59 self-test covers

- Requirement statuses for all three synthetic files, adjusted for installed optional libraries.

- Graceful skipping of a corrupt file, and batch continuation past it.

- --fail-fast exit code 1, and roll-up HTML and CSV generation.

- Version stamp consistency across the package, VERSION.txt, the launcher filename, the JSON schema_version, and --version.

- Req 4 returning PASS when no optional extension is registered.

- --quiet emitting no report-written chatter while still printing the per-file summary line.

- Every manual-check entry being a uniform 4-tuple, verified by an actual unpack.

- The conformance-scope banner being present in per-file and roll-up HTML.

- Req 18 stating the XSD validation outcome, and lxml being present in the optional-library probe.

- Release assets — QUICKSTART.html, the v1.59 manual, package_release.py, the EPSG cache — being present.

## 9.3 Building the release archive

Once the self-test passes, build the distributable archive. The script is standard library only, so it runs in any environment:

python package_release.py

Output: dist\DGIWG_GeoPackage_Validator_v1.59.zip, with a staged folder of the same name beside it and a generated VERSION.txt inside. The script verifies a manifest of required assets first and aborts without writing anything if one is missing.

# 10. Known Limitations

- Requirements 1 and 2 (OGC base conformance) are not automated — use OGC CITE TeamEngine.

- Requirement 6 (conditional extensions) depends on the applicable DGIWG product profile and is always SKIPPED.

- Some checks return PASS* when their optional library (pyproj, shapely, Pillow or lxml) is not installed. Install all four for the most complete automated verification.

- Scale-denominator verification is not available for UTM and Lambert Conformal Conic raster CRS, because no single global pixel sequence exists for them. Those return PASS* on Req 7 with the CRS confirmed as allowed.

- Req 24 and Req 26 sample BLOBs rather than scanning exhaustively — 25 geometries per table and 5 tile BLOBs per zoom level by default. Raise --sample-size for deeper coverage; a full-scan mode is not yet available.

- Req 20 and Req 23 are capped at PASS* by design: metadata completeness can be confirmed structurally but not judged semantically by machine.

- The forensic source-software detection is evidence-based and reports a confidence level. Treat an UNKNOWN result as “not determined”, not as “suspicious”.

# 11. Support

Mapping and Charting Establishment / Geomatics Engineering Trials & Evaluation Support Section (GETESS).

Contact: euisoo.son@forces.gc.ca

When reporting a problem, include the validator version (python -m dgiwg_validator --version), the Python version, the output of run_local_tests.py together with its log file, and — where it can be shared — the JSON companion report for the file in question. Those four items are usually enough to reproduce and diagnose an issue without the GeoPackage itself.

DGIWG GeoPackage Compliance Validator v1.59  —  GPL-2.0-or-later  —  © 2026 Eui Soo SON