GeoPackage Creator

Test Concept & Execution Plan

Data Variety Testing for DGIWG / OGC GeoPackage Compliance

Version: v0.29.1 ¦ Date: 2026-06-16 ¦ GeoPackage Creator Project

# 1. Overall Test Concept Diagram

Step 1 — Collect & Prepare Test Data

▼

| Vector (Basic) SHP / GDB Point · Line · Polygon | Vector (Advanced) Large-scale · Multi-layer Z/M coordinates | Raster (Elevation) DTED / DEM Copernicus GLO-30 | Raster (Imagery) Sentinel-2 NAIP aerial photos |
| --- | --- | --- | --- |

▼

Step 2 — Run GeoPackage Creator Conversion

▼

| Phase 1 Basic Conversion • CRS auto-detection • Mode A / B / C • HTML + JSON report • Layer & feature count | Phase 2 DGIWG Compliance • Req 3 Application ID • Req 13 WKT2 CRS • Req 24 Z/M check • Req 24 BBox match | Phase 3 Raster & Edge Cases • Tile pyramid • Gridded elevation (Req 33-37) • NULL / empty geometry • Special-char layer names |
| --- | --- | --- |

▼

Step 3 — DGIWG Validator Automated Check

▼

| CONFORMANT All DGIWG requirements passed |  | NON-CONFORMANT Analyse failed Req -> Fix -> Retest |
| --- | --- | --- |

# 2. Test Data Catalogue

## 2-1. Vector Data

| Priority | Data Source | Format / Characteristics | Test Focus |
| --- | --- | --- | --- |
| 1st | Natural Earth 110m | SHP — Point/Line/Polygon | Geometry type variety; ideal for first test |
| 2nd | GADM Canada Level 2 | GDB — 20+ layers | Multi-layer depth, complex attributes |
| 3rd | Geofabrik OSM (Canada) | SHP — 100 MB+ | Large-scale performance & memory handling |
| Aux | NRN (National Road Network) | GDB — defence road data | DGIWG schema mapping fitness |
| Aux | Statistics Canada | SHP — census boundaries | Attribute encoding (UTF-8 / CP1252) |

## 2-2. Raster Data  (for v0.28+ readiness)

| Type | Data Source | Resolution / Characteristics | DGIWG Requirement |
| --- | --- | --- | --- |
| Elevation | Copernicus DEM GLO-30 | 30 m — DTED Level 2 substitute | Req 33-37 (Gridded Coverage) |
| Elevation | SRTM 1-arc-second | 30 m — NASA / EarthExplorer | Req 11-12 (Gridded CRS) |
| Image Tile | Sentinel-2 L2A | 10 m — ESA Copernicus Hub | Req 7-8, 25-31 (Tile Matrix) |
| Image Tile | NAIP Aerial | 1 m — high-res USA | High-res tile pyramid validation |

## 2-3. Edge Cases  (mandatory)

| Edge Case | Risk | Related DGIWG Req |
| --- | --- | --- |
| Z/M coordinate geometry | Misread Z/M flag bits -> false FAIL | Req 24 -- Fixed in v0.29 |
| Empty / NULL geometry | BBox calculation crash | Req 24 (BBox match) |
| GDB with 20+ layers | Memory / performance bottleneck | Req 3 (Application ID) |
| Layer names with special chars | SQL name collision, encoding error | Req 3 |
| Non-approved CRS (EPSG:3857) | CRS rejection logic silent | Req 13 (WKT2 CRS) |
| Mixed geometry (Polygon + Multi) | Geometry type mismatch error | Req 24 |

# 3. Phase-by-Phase Execution Guide

## Phase 1  —  Basic Conversion  (small vector datasets)

Goal: Verify that GeoPackage Creator correctly converts basic vector data and generates DGIWG-compliant reports.

| Step | Task | Details | Verification |
| --- | --- | --- | --- |
| 1 | Prepare data | Download Natural Earth 110m SHP from naturalearthdata.com -> 'Free vector' | Confirm SHP + DBF + PRJ set exists |
| 2 | Launch GUI | Open GeoPackage Creator GUI, select source file, set output path | Application starts without error |
| 3 | Enter metadata | Fill Title, Abstract, POC, Org fields; select CRS Mode A (auto-convert) | Warning shown if required fields are empty |
| 4 | Run conversion | Click Convert; monitor real-time progress log | No errors; .gpkg file created at output path |
| 5 | Review report | Open HTML report in browser; inspect JSON report | success: true; layer count and feature count correct |
| 6 | DGIWG validation | Run DGIWG Validator against the .gpkg file | All 37 requirements CONFORMANT |

## Phase 2  —  DGIWG Compliance Deep-Dive

Goal: Systematically verify each DGIWG requirement by rotating data types and CRS inputs.

| Step | Task | Details | Verification |
| --- | --- | --- | --- |
| 1 | CRS variety | Convert UTM, NAD83, and WebMercator datasets via Mode A -> confirm EPSG:4326 output | Req 13: WKT2 CRS registration PASS |
| 2 | Z/M geometry | Convert data containing 3D (Z) geometry; inspect gpkg_geometry_columns | Req 24: Z=0 layers contain no Z geometry |
| 3 | BBox check | Convert large dataset (Geofabrik); compare gpkg_contents bbox vs RTree aggregate | Req 24: exact bbox match |
| 4 | Metadata | Validate DMF metadata XML; inspect gpkg_metadata table entries | Req 3 & Req 31: metadata completeness |
| 5 | Rejected CRS | Attempt conversion with EPSG:3857 source without Mode A | Req 13: clear rejection message displayed |
| 6 | Edge cases | Test empty geometry, NULL values, special-char layer names in sequence | No crash; informative error messages |

## Phase 3  —  Raster Support  (v0.28+ readiness)

Goal: After raster support is added, validate tile and elevation data against DGIWG Req 7-8, 11-12, 25-37.

| Step | Task | Details | Verification |
| --- | --- | --- | --- |
| 1 | Prepare DEM | Download Copernicus GLO-30 GeoTIFF; clip to area of interest using gdal_translate | GeoTIFF loads cleanly in GDAL |
| 2 | Tile conversion | Convert DEM to GPKG elevation tiles; 256x256 tile matrix, zoom levels 0-10 | Req 25-30: tile matrix conformant |
| 3 | CRS validation | Tile CRS: EPSG:4326 or UTM; polar data: UPS EPSG:5041/5042 | Req 7, Req 11-12 PASS |
| 4 | Image tiles | Convert Sentinel-2 to GPKG image tiles; verify PNG/JPEG tile format | Req 16-17 (tile format) PASS |
| 5 | Combined test | Create GPKG with both vector and raster layers; run DGIWG Validator | All 37 requirements PASS |

# 4. Briefing / Presentation Guide

## 4-1. Recommended Session Structure  (60 minutes)

| Session | Time | Content |
| --- | --- | --- |
| Opening | 5 min | What is GeoPackage? Why does DGIWG compliance matter? -- 1-slide summary for non-technical audiences |
| Tool Demo | 10 min | Live GUI demo with Natural Earth data -- run one conversion end-to-end in front of audience |
| Concept Diagram | 15 min | Walk through Sections 2-3 of this document -- 3 phases, data types, why edge cases matter |
| Live Validation | 15 min | Run DGIWG Validator live; share screen showing CONFORMANT result HTML report |
| Roadmap | 10 min | Phase 3 raster support plan, additional data sources, open Q&A |
| Q&A | 5 min | Questions and discussion on future collaboration |

## 4-2. Key Messages by Audience

| Audience | Core Message |
| --- | --- |
| GIS Developers | Which code changed for each DGIWG Req; how validation is automated; raster architecture design |
| GIS Analysts | What data goes in, what GeoPackage comes out -- focus on I/O and recommended data sources |
| Defence / Policy | Automated compliance against all 37 DGIWG requirements; guaranteed NATO operational interoperability |
| Non-technical Mgmt | "Put a file in -> get an international-standard GeoPackage out" -- concept diagram + live demo only |

## 4-3. Pre-Briefing Checklist

| Check | Preparation Item |
| --- | --- |
| [ ] | Download Natural Earth SHP in advance -- demo must work without internet |
| [ ] | Confirm GeoPackage Creator GUI launches and all dependencies are installed |
| [ ] | Confirm DGIWG Validator runs successfully in the Python environment |
| [ ] | Pre-convert one .gpkg sample file as a demo backup in case live conversion fails |
| [ ] | Open the HTML report in a browser tab before the session begins |
| [ ] | Prepare concept diagram on large screen or as printed handout |
| [ ] | Prepare a separate handout listing all recommended data source URLs |

# 5. Executive Summary

| Item | Summary |
| --- | --- |
| Current Status | Full vector conversion support; all 37 DGIWG requirements pass automatically (v0.29.1) |
| Ready to Test Now | Proceed in order: Natural Earth -> GADM -> Geofabrik -> edge cases |
| Pending | Raster support (v0.28+): collect Copernicus DEM and Sentinel-2 data in advance |
| Edge Case Priority | Z/M geometry -> empty geometry -> mixed geometry -> special-char layer names |
| Briefing Essentials | Concept diagram + live demo + DGIWG validator result -- three elements are sufficient |

This document is based on GeoPackage Creator v0.29.1. Update Section 3 Phase 3 when raster support is released.